University of Nebraska – Lincoln

Department of Psychology

Canine Cognition and Human Interaction Laboratory

Dr. Jeffrey R. Stevens

Precise Dispenser for Canine Numerical Discrimination Tasks

14 September 2021

V0.3

Figure 1 – Dispenser v84 Render

Overview

The performance of canine numerical discrimination tasks can be hindered by a lack of appropriate instrumentation.  Using manual, researcher-controlled instrumentation can introduce a reaction time variable that could impose undue variability in the task results.  Therefore, the creation of devices that can automatically dispense precise numbers of treats at the introduction of a predefined stimulus can be justified.

Dispenser Operation

The precise dispenser has a wheel on the top of it that holds up to 59 treats.  The stepper motor shaft interfaces to the dispenser wheel through the center hole and by actuating the motor, the holes for the treat lines up with the treat exhaust and falls to the bowl below.  When the treat goes through the exhaust it breaks an infrared break beam and the Raspberry Pi detects the treat dispensing.

Dispenser Design

The precise dispenser comprises five major components: the treat jogger, the cover, the top plate, the walls, and the bottom plate.  Each piece was designed in Autodesk Fusion 360 (Autodesk 2021).  The version documented in this paper is v84.

Figure 1 - (Left) Overhead view of the treat jogger and (right) a detail view of the set screw

The treat jogger consists of a 3D printed wheel with two circles of receptacles for treats, leading to a total treat capacity of 59 treats.  The shaft of a stepper motor is set in the center of the wheel, with a set screw being used to solidly affix the wheel to the motor shaft.

Figure 2 - Dispenser cover with a center depression for an 8-inch acrylic disk

The cover goes over the top of the dispenser and seals the treats from the canine in the experiment, there is a tab on the backside of the cover to allow is to be removed from the dispenser.

Figure 3 - (Left) Overhead view of the top plate of the dispenser and (right) a detailed view showing the two lips of the plate

The top plate of the dispenser has the stepper motor mounted to it and the top of the pellet exhaust exposed, which is elongated to allow either the top or bottom row treat to be dispensed.  The top of this plate has a lip to press fit the dispenser cover and the bottom of this plate has a lip to press fit the walls and complimentary M3 screw holes to mechanically attach the two pieces.

Figure 4 – (Left) The walls of the dispenser which has the connectors and electronics mounted to it and (right) a detail shot of the mounting point for the IR break beam sensor

The walls of the dispenser have the electronics mounted to it and exposes the connectors for power and data.  The pellet exhaust can be seen as well as the mounting point for the IR break beam sensor, which has a complimentary detector on the opposite side of the pellet exhaust.  Each of the bottom mounting points for the bottom plate have an M3 nut capture to allow the bottom plate to be screwed on without holding the nut in place.

Figure 5 - Connector panel detail view: (a) global power switch, (b) 12V motor power supply, (c) 5V Raspberry Pi power supply, (d) HDMI panel mount connector, (e) exposed Raspberry Pi 4 connectors

The connector panel of the dispenser exposes all connection points necessary to interact with the system.  The two power supplies can be disconnected from the system using the DPDT power switch, a display can be connected through the HDMI connector, and USB and Ethernet can be accessed directly on the Raspberry Pi.

Figure 6 - Bottom plate of the dispenser with feet to stand it higher off the main surface

The bottom plate of the dispenser closes the system up and isolates the electronics from the main experiment area.  Eleven M3 screws are used to secure it to the walls of the dispenser.  This is also the access point for service to the electronics.

Circuit Descriptions

Python Class Definition

class PreciseDispenser

__init__(loaded_treats, timeout=1)

Initializer for the canine treat dispenser, configures control variables and Pi GPIO.

:param loaded_treats: The number of treats that are loaded into the dispenser for this session.  Defaults to max, 59 treats.

:param timeout: The amount of time before the dispenser throw an error between dispensation events

:param version: The HAT version that is being used, "BB" for breadboard and "P" for production

:returns: None.

dispense_treat()

Function to dispense a single treat.  Should not be used, use dispense_treats for error handling.

:returns: True for successful dispensation of a single treat, False for a failure to dispense the treat.

dispense_treats(num_treats)

Dispenses the specified number of treats.  If the requested number of treats is greater than the remaining treats, a ValueError is thrown.  If the treat dispenser loop times out, a ValueError is thrown.

:param num_treats: A number between 1 and the remaining number of treats.

:returns: None.  Throws ValueError when an error event is encountered.	treat_dispensed(channel):

A callback function for when a treat is dispensed.  This is detected on the falling edge of the IR break beam.

:param channel: The channel to identify the callback source.

:returns: None.

close()

Closes the dispenser object and cleans up the GPIO assignments.

:returns: None.

Interfacing Example

from precise_dispenser_driver import PreciseDispenser

dispenser = PreciseDispenser()

try:

dispenser.dispense_treats(10)

print(“Successfully dispensed ten treats.”)

except ValueError as e:

print(e)

dispenser.close()

Raspberry Pi 4 as an Experiment Platform

The experiments on canine operant conditioning are run using a Python package called PsychoPy.  It essentially acts as a development environment for psychology experiments and provides a variety of utilities for user interface development, trigger conditions, experiment logging, and so on.  Our aim is to integrate this directly into a Raspberry Pi 4, to allow the entire setup to become a mobile testing unit.  To do this, we will have a few requirements:

- Wireless Access Point

  - The Raspberry Pi 4 should host its own WiFi access point (AP), which allows the experimenter to connect to the Pi and run experiments, download results, and interact with the main system.

- Decoupled Interfaces

  - The screen the experimenter will use, and the screen used by the canine will be separate and require no extra hardware to achieve.  By connecting to the Pi using SSH (Secure Shell), the command line can be exposed to run the experiments.  The canine screen can be connected to the HDMI0 of the Pi and the SSH session can be started in a separate area.

- Simple Setup

  - We want to make sure this is easy to setup and maintain.  This means that we would want to use robust packages such as ‘pi-ap’ and SD card backups to easily restore the device and get new devices up and running easily.

There are two ways to setup a new Raspberry Pi to operate the canine dispenser:

- Flash a new SD card with an image from the iso_images folder

  - Using a Raspberry Pi with a fresh installation of Raspbian Buster, there are built-in tools to achieve this.  “The SD Card Copier application, which can be found on the Accessories menu of the Raspberry Pi Desktop, will copy Raspbian from one card to another. To use it, you will need a USB SD card writer.  To back up your existing Raspbian installation, put a blank SD card in your USB card writer and plug it into your Pi, and then launch SD Card Copier. In the ‘Copy From Device’ box, select the internal SD Card. This could have a number of different names and may have something like (/dev/mmcblk0) in its entry, but will usually be the first item in the list. Then select the USB card writer in the ‘Copy To Device’ box (where it will probably be the only device listed). Press ‘Start’. The copy, depending on the size of the SD card, can take ten or fifteen minutes, and when complete you should have a clone of your current installation on the new SD card. You can test it by putting the newly-copied card into the Pi’s SD card slot and booting it; it should boot and look exactly the same as your original installation, with all your data and applications intact.”

- Follow the proceeding instructions to setup the Raspberry Pi manually.

  - Flash a new copy of Rasbian Buster onto a micro SD card and place an empty file named “ssh” onto the boot partition.  There is no file extension.

  - Connect the Raspberry Pi to your local network using an Ethernet cable and power up the Pi.  You can connect a monitor, keyboard, and mouse and use ifconfig to find the IP address of the Pi.

  - Using this IP address, go to PuTTY and type in this IP address and connect to the Pi.  The default password for “pi” is “raspberry”.

  - Once in, the pi-ap software can be installed to create the access point.  Note, the dog_operant repository does contain a copy of this, but the main repository will contain the most recent version.

  - The dog_operant repository can be cloned using the git clone command.

  - Run the command sudo apt-get update

  - You will need to upgrade pip using 
python3 -m pip install --upgrade pip 
which will allow it to index some of the following packages.

  - Run the command 
python3 -m pip install psychopy==3.2.4 
which will install all the needed packages.  Using the older version of PsychoPy will prevent a ‘Segmentation fault’ caused when rendering a new Window.

  - You may have errors from installing PyQt5, to remedy this, try the following:
sudo apt-get update

sudo apt-get install qt5-default pyqt5-dev pyqt5-dev-tools

  - You may have errors importing Pandas due to an Import Error from NumPy, this can be fixed by doing:
sudo apt-get install libatlas-base-dev

  - You may get an error saying that there is no package called ‘wx’, in which case you need to run the command 
python3 -m pip install wxPython

  - Step k may also error out, so issue the following command and then perform step k again:
sudo apt-get install dpkg-dev build-essential libjpeg-dev libtiff-dev libsdl1.2-dev libgstreamer-plugins-base0.10-dev libnotify-dev freeglut3 freeglut3-dev libwebkitgtk-dev libghc-gtk3-dev libwxgtk3.0-gtk3-dev

  - To install the audio tools, issue the following commands:
sudo apt-get install portaudio19-dev

python3 -m pip install pysine

  - To install the serial tools, issue the following command:
python3 -m pip install pyserial

  - If you want to run experiments, you will need to connect a screen to the Pi on HDMI0.  An issue may present itself where no screen is presented when connected, which is caused by an incorrect /boot/config.txt file.  To fix this do the following:

    - sudo nano /boot/config.txt

    - Add the following to the file or uncomment from the file,

      - hdmi_force_hotplug=1

      - hdmi_drive=2

    - If this does not fix the issue, add,

      - hdmi_safe=1

  - To run an experiment, use the following command 
export DISPLAY=:0 ; python3 dog_operant/system_test.py 
Note, this works from the command line, so run this when you are connected to an SSH session and with a screen connected to HDMI0 of the Pi and the experiment will display over HDMI0.  Setting the DISPLAY variable to 0 will default it to the local display.

- To transfer the results of the experiments, copy the contents of the data directory from the dog_operant directory.  Use the following command structure: 

scp -r user@ssh.example.com:/path/to/remote/source /path/to/local/destination

Raspberry Pi Interfacing and Passwords

The dispenser, when turned on, will create a WiFi access point called, “cchil-precise-stevens-x”, with “x” being the number, i.e. “1”.  By connecting to this, the Raspberry Pi can be accessed through SSH using a program called PuTTY.

| Local Address | Password |
| --- | --- |
| pi@192.168.0.1 | raspberry |
| pi@cchil-precise-stevens-1 | raspberry |

Figure 2 – PuTTY Terminal Configuration Window

Reliability Testing

Bill of Materials

| Bill of Materials - Precise Dispenser |  |  |  |
| --- | --- | --- | --- |
| Part | Quantity | Cost | Link |
| 1 kg black filament | 1 | $ 22.99 | Filament |
| Raspberry Pi 4 | 1 | $ 55.00 | Raspberry Pi |
| HDMI Panel Mount | 1 | $ 7.95 | HDMI |
| USB C Panel Mount | 1 | $ 7.50 | USB C |
| Barrel Jack Panel Mount | 1 | $ 5.17 | Barrel Jack |
| HDMI to Micro HDMI | 1 | $ 8.69 | Micro HDMI |
| Stepper Motor Power Supply | 1 | $ 11.61 | 12V Power |
| Stepper Motor | 1 | $ 8.99 | Motor |
| Raspberry Pi Power Supply | 1 | $ 8.00 | Pi Power |
| Plastic Cover | 1 | $ 9.95 | 8 in Cover |
| IR Break Beam | 1 | $ 1.95 | 3mm IR |
| M3 Screws | 1 | $ 26.99 | M3 Pack |
| M3 Hex Wrench | 1 | $ 14.99 | Hex Set |
| Cable Ties | 1 | $ 9.69 | Zip Ties and Mounts |
| Micro SD Card | 1 | $ 9.49 | SD Card Pack |
| USB C Cable | 1 | $ 6.99 | USB C Cables |

| Bill of Materials - Precise Dispenser Pi HAT |  |  |  |
| --- | --- | --- | --- |
| Part | Quantity | Cost | Link |
| PCB | 3 | $ 28.45 | [Add OSHPark Link] |
| A4988 Stepper Motor | 1 | $ 4.69 | Motor Driver |
| 2 Pin Screw Terminal | 2 | $ 2.20 | 2x 3.5mm Terminal |
| 3 Pin Screw Terminal | 1 | $ 1.35 | 3x 3.5mm Terminal |
| Capacitor | 1 | $ 0.13 | 10u TH Cap |
| Resistor | 1 | $ 0.10 | 10k TH Res |
| 40 Pin Header | 1 | $ 0.95 | Pi Header |
| Stepper Motor Header | 1 | $ 0.20 | 4 Pin Right Angle |
| A4988 Female Pins | 2 | $ 1.30 | 2x Female Headers |

Build Instructions

Document Revision History

| Revision, Name, and Date | Revision Description |
| --- | --- |
| V0.1, Walker Arce, 7 March 2021 | First draft creation date for the precise dispenser |
| V0.2, Walker Arce, 3 April 2021 | Added Python driver documentation |
| V0.3, Walker Arce, 9 April 2021 | Added Python interfacing example and bills of material |